Centrifugal fans are mechanical devices that move air or other gases at a 90-degree angle from the intake, using centrifugal force to increase both pressure and velocity. They are a critical component in systems that must overcome high resistance, such as those with extensive ductwork or complex filtration.

Description and Working Principle

A centrifugal fan consists of a rotating impeller (fan wheel) housed within a scroll-shaped casing or volute.

- Intake: Air enters the fan axially (parallel to the rotation axis) through the "eye" or center of the impeller.

- Acceleration: As the motor spins the impeller, the blades catch the air and accelerate it outward toward the casing using centrifugal force.

- Pressure Conversion: As the high-speed air leaves the impeller and enters the wider areas of the volute casing, its kinetic energy is converted into static pressure.

- Discharge: The pressurized air is then expelled through the outlet, perpendicular to the original intake direction.

Key Benefits

- High Pressure Capability: Unlike axial fans, centrifugal fans can generate significant static pressure, making them ideal for pushing air through filters, dampers, and long ducts.

- Exceptional Efficiency: Modern designs, particularly those with backward-curved or airfoil blades, can achieve static efficiencies of 80% to 90%.

- Durability and Robustness: Often constructed from heavy-duty materials like stainless steel or specialized alloys, they are designed to handle high temperatures (up to 400°C+), corrosive gases, and abrasive dust.

- Non-Overloading Characteristics: Many models feature "self-limiting" horsepower curves, ensuring the motor will not burn out even if the airflow capacity is exceeded.

- Versatility: They are highly customizable in terms of size, blade type (forward, backward, or radial), and drive mechanism (direct or belt drive) to meet specific industrial needs.

Common Applications

- HVAC Systems: Used for air circulation and temperature regulation in large commercial buildings and data centers.

- Industrial Ventilation: Essential for removing smoke, toxic fumes, and vapours in factories, mines, and laboratories.

- Material Handling: Capable of conveying bulk solid materials such as grain, sand, wood chips, and plastic pellets through pipelines.

- Air Pollution Control: Integrated into baghouses, scrubbers, and cyclones to filter particulate matter before air is released into the environment.

- Cooling and Drying: Employed in food processing, textiles, and power plants for drying products or cooling sensitive machinery like boilers and kilns.